SECTION 1: Identification

1.1. Product identifier

| Product name | : | ALL PURPOSE CLEAN NATURAL |
| --- | --- | --- |
| Product code | : | A300000 |

1.2. Recommended use and restrictions on use

| Recommended use | : | All-purpose cleaner |
| --- | --- | --- |
| Restrictions on use | : | Food Plant, Industrial and Institutional use only |

1.3. Supplier

Project Clean Inc. 12 James St N, Suite 202 Hamilton, Ontario L8R 2J9 T 1 800 663 9925 www.projectclean.ca

1.4. Emergency telephone number

| Country | Organisation/Company | Address | Emergency number | Comment |
| --- | --- | --- | --- | --- |
| Canada | CHEMTREC Chemical Emergency | www.chemtrec.com | 1 800 424 9300 | 24hr/day 7days/week within USA and Canada |
| Canada | CANUTEC Transportation Emergency | www.canutec.com | 1 888 226 8832 *666 on a cell phone | 24hr/day 7days/week within USA and Canada |

SECTION 2: Hazard identification

2.1. Classification of the substance or mixture

Classification (GHS CA)

| Skin corrosion/irritation, Category 2 | H315 | Causes skin irritation. |
| --- | --- | --- |
| Serious eye damage/eye irritation, Category 1 | H318 | Causes serious eye damage. |

Full text of H-statements: see section 16

2.2. GHS Label elements, including precautionary statements

GHS CA labelling

| Hazard pictograms (GHS CA) | : |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Signal word (GHS CA) | : | Danger |  |  |  |  |  |

| Hazard statements (GHS CA) | : | H315 - Causes skin irritation H318 - Causes serious eye damage |
| --- | --- | --- |
| Precautionary statements (GHS CA) | : | P264 - Wash hands, forearms and face thoroughly after handling. P280 - Wear protective gloves, protective clothing, and eye protection. P302+P352 - IF ON SKIN: Wash with plenty of water. P305+P351+P338 - IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing. P310 - Immediately call a POISON CENTER or a doctor. P321 - Specific treatment (see supplemental first aid instruction on the SDS). P332+P313 - If skin irritation occurs: Get medical advice or attention. P362+P364 - Take off contaminated clothing and wash it before reuse. |

2.3. Other hazards

| Other hazards which do not result in classification | : | Harmful to aquatic life with long lasting effects. |
| --- | --- | --- |

2.4. Unknown acute toxicity (GHS CA)

No data available

SECTION 3: Composition/information on ingredients

3.1. Substances

Not applicable

3.2. Mixtures

| Name | Chemical name / Synonyms | Product identifier | % w/w |
| --- | --- | --- | --- |
| Alkyl Glucoside | D-Glucopyranose, oligomers, decyl octyl glycosides | CAS-No.: 68515-73-1 | 5 – 10 |
| N,N-dimethyldodecylamine-N-oxide | N,N-Dimethyl-1-dodecanamine, N-oxide | CAS-No.: 1643-20-5 | 1 – 5 |
| Glycolipids, sophorese-contg, Stamerella bombicola-fermented, from cooking oil wastes and sucrose | Glycolipids | Proprietary | 1 – 5 |
| Citric acid | 2-hydroxypropane-1,2,3-tricarboxylic acid | CAS-No.: 77-92-9 | 1 – 5 |
| N,N-dimethyltetradecylamine N-oxide | 1-tetradecanamine, N,N-dimethyl-, N-oxide | CAS-No.: 3332-27-2 | 0,5 – 1,5 |
| Sodium hydroxide | Sodium hydroxide | CAS-No.: 1310-73-2 | 0,5 – 1,5 |

*The exact concentrations have been withheld as a trade secret. Les concentrations exactes ont été retenues en tant que secret commercial.

Full text of hazard classes and H-statements : see section 16

SECTION 4: First-aid measures

4.1. Description of first aid measures

| First-aid measures after inhalation | : | Remove person to fresh air and keep comfortable for breathing. |
| --- | --- | --- |
| First-aid measures after skin contact | : | Wash skin with plenty of water. Take off contaminated clothing. If skin irritation occurs: Get medical advice or attention. |
| First-aid measures after eye contact | : | Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing. Call a physician immediately. |
| First-aid measures after ingestion | : | Call a poison center or a doctor if you feel unwell. |
| First-aid measures general | : | If you feel unwell, seek medical advice. |

4.2. Most important symptoms and effects (acute and delayed)

| Symptoms/effects after inhalation | : | None under normal conditions. |
| --- | --- | --- |
| Symptoms/effects after skin contact | : | Irritation. |
| Symptoms/effects after eye contact | : | Serious damage to eyes. |
| Symptoms/effects after ingestion | : | None under normal conditions. |
| Chronic symptoms | : | No effects known. |
| Expected Symptoms/Effects, Acute and Delayed | : | May cause dermatitis, eye irritation, corneal oedema and chemical burns. Repeated or prolonged contact may cause skin irritation. |

4.3. Immediate medical attention and special treatment, if necessary

| Other medical advice or treatment | : | Treat symptomatically. |
| --- | --- | --- |

SECTION 5: Fire-fighting measures

5.1. Suitable extinguishing media

| Suitable extinguishing media | : | Water spray. Dry powder. Foam. Carbon dioxide. |
| --- | --- | --- |

5.2. Unsuitable extinguishing media

| Unsuitable extinguishing media | : | Do not use a heavy water stream. |
| --- | --- | --- |

5.3. Specific hazards arising from the hazardous product

| Fire hazard | : | No fire hazard. |
| --- | --- | --- |
| Explosion hazard | : | No direct explosion hazard. |
| Hazardous decomposition products in case of fire | : | Toxic fumes may be released. |

5.4. Special protective equipment and precautions for fire-fighters

| Firefighting instructions | : | Fight fire from safe distance and protected location. Do not enter fire area without proper protective equipment, including respiratory protection. |
| --- | --- | --- |
| Protection during firefighting | : | Do not attempt to take action without suitable protective equipment. Self-contained breathing apparatus. Complete protective clothing. |

SECTION 6: Accidental release measures

| General measures | : | Stop leak if safe to do so. Notify authorities if product enters sewers or public waters. Absorb spillage to prevent material damage. |
| --- | --- | --- |

6.1.1. For non-emergency personnel

| Protective equipment | : | Chemical goggles or face shield with safety glasses. |
| --- | --- | --- |
| Emergency procedures | : | Ventilate spillage area. Evacuate unnecessary personnel. Avoid contact with skin and eyes. |

6.1.2. For emergency responders

| Protective equipment | : | Do not attempt to take action without suitable protective equipment. For further information refer to section 8: "Exposure controls or personal protection". |
| --- | --- | --- |
| Emergency procedures | : | Ventilate area. Prevent from entering sewers, basements and workpits, or any place where its accumulation can be dangerous. Evacuate unnecessary personnel. Stop leak if safe to do so. |

6.2. Environmental precautions

Avoid release to the environment.

6.3. Methods and material for containment and cleaning up

| For containment | : | Absorb spilled material with sand or earth. Contain any spills with dikes or absorbents to prevent migration and entry into sewers or streams. Stop leak without risks if possible. |
| --- | --- | --- |
| Methods for cleaning up | : | Take up liquid spill into absorbent material. |
| Other information | : | Dispose of materials or solid residues at an authorized site. |

SECTION 7: Handling and storage

7.1. Precautions for safe handling

| Precautions for safe handling | : | Ensure good ventilation of the work station. Avoid contact with skin and eyes. Wear personal protective equipment. |
| --- | --- | --- |
| Hygiene measures | : | Wash contaminated clothing before reuse. Do not eat, drink or smoke when using this product. Always wash hands after handling the product. |
| Additional hazards when processed | : | Not expected to present a significant hazard under anticipated conditions of normal use. |

7.2. Conditions for safe storage, including any incompatibilities

| Technical measures | : | Keep in a cool, well-ventilated place away from heat. |
| --- | --- | --- |
| Storage conditions | : | Keep cool. Protect from sunlight. |
| Incompatible products | : | Strong oxidizing agents. Strong reducing agents. |
| Packaging materials | : | Store always product in container of same material as original container. |

SECTION 8: Exposure controls/personal protection

8.1. Control parameters

| Sodium hydroxide (1310-73-2) |
| --- |
| Canada (Alberta) - Occupational Exposure Limits |  |
| Local name | Sodium hydroxide |
| OEL C | 2 mg/m³ |
| Notations and remarks | Occupational exposure limit is based on irritation effects and its adjustment to compensate for unusual work schedules is not required. |
| Regulatory reference | Alberta Regulation 191/2021 |
| Canada (Quebec) - Occupational Exposure Limits |  |
| Local name | Sodium hydroxide |
| Plafond (OEL C) | 2 mg/m³ |
| Notations and remarks | RP |
| Regulatory reference | S-2.1, r. 13 - Regulation respecting occupational health and safety |
| Canada (British Columbia) - Occupational Exposure Limits |  |
| Local name | Sodium hydroxide |
| OEL C | 2 mg/m³ |
| Regulatory reference | OHS Guidelines Part 5: Chemical Agents and Biological Agents (WorkSafe BC) |
| Canada (Manitoba) - Occupational Exposure Limits |  |
| Local name | Sodium hydroxide |
| OEL C | 2 mg/m³ |
| Notations and remarks | TLV® Basis: URT, eye, & skin irr |
| Regulatory reference | ACGIH 2022 |
| Canada (Newfoundland and Labrador) - Occupational Exposure Limits |  |
| Local name | Sodium hydroxide |
| OEL C | 2 mg/m³ |
| Notations and remarks | TLV® Basis: URT, eye, & skin irr |
| Regulatory reference | ACGIH 2022 |
| Canada (Nova Scotia) - Occupational Exposure Limits |  |
| Local name | Sodium hydroxide |
| OEL C | 2 mg/m³ |
| Notations and remarks | TLV® Basis: URT, eye, & skin irr |
| Regulatory reference | ACGIH 2022 |
| Canada (Nunavut) - Occupational Exposure Limits |  |
| Local name | Sodium hydroxide |
| OEL C | 2 mg/m³ |
| Regulatory reference | Occupational Health and Safety Regulations, Nu Reg 003-2016 (Amendment R-044-2021) |
| Canada (Northwest Territories) - Occupational Exposure Limits |  |
| Local name | Sodium hydroxide |
| OEL C | 2 mg/m³ |
| Regulatory reference | Occupation Health and Safety Regulations R-039-2015 (R-013-2020) |
| Canada (Ontario) - Occupational Exposure Limits |  |
| Local name | Sodium hydroxide |
| OEL C | 2 mg/m³ |
| Regulatory reference | Ontario Occuational Exposure Limits under Regulation 833 |
| Canada (Prince Edward Island) - Occupational Exposure Limits |  |
| Local name | Sodium hydroxide |
| OEL C | 2 mg/m³ |
| Notations and remarks | TLV® Basis: URT, eye, & skin irr |
| Regulatory reference | ACGIH 2022 |
| Canada (Saskatchewan) - Occupational Exposure Limits |  |
| Local name | Sodium hydroxide |
| OEL C | 2 mg/m³ |
| Regulatory reference | The Occupational Health and Safety Regulations, 2020. Chapter S-15.1 Reg 10 |
| USA - ACGIH - Occupational Exposure Limits |  |
| Local name | Sodium hydroxide |
| ACGIH® TLV® C | 2 mg/m³ |
| Remark (ACGIH) | TLV® Basis: URT, eye, & skin irr |
| Regulatory reference | ACGIH 2022 |
| USA - OSHA - Occupational Exposure Limits |  |
| Local name | Sodium hydroxide |
| OSHA PEL TWA | 2 mg/m³ |
| Regulatory reference (US-OSHA) | OSHA Annotated Table Z-1 |

8.2. Appropriate engineering controls

| Appropriate engineering controls | : | Ensure good ventilation of the work station. |
| --- | --- | --- |
| Environmental exposure controls | : | Avoid release to the environment. |

8.3. Individual protection measures/Personal protective equipment

Personal protective equipment:

Wear recommended personal protective equipment.

Materials for protective clothing:

Nitrile rubber/PVC

Hand protection:

Protective gloves against chemicals (EN 374)

Eye protection:

Safety glasses

Skin and body protection:

Protective clothing

Respiratory protection:

In case of insufficient ventilation, wear suitable respiratory equipment

Personal protective equipment symbol(s):

SECTION 9: Physical and chemical properties

9.1. Information on basic physical and chemical properties

| Physical state | : | Liquid |
| --- | --- | --- |
| Appearance | : | Translucent. |
| Colour | : | Tan colour |
| Odour | : | No added fragrance |
| Odour threshold | : | No data available |
| pH | : | 6 – 8 |
| Relative evaporation rate (butylacetate=1) | : | No data available |
| Relative evaporation rate (ether=1) | : | No data available |
| Melting point | : | Not applicable |
| Freezing point | : | No data available |
| Initial boiling point and boiling range | : | No data available |
| Flash point | : | No data available |
| Auto-ignition temperature | : | Not self-igniting |
| Decomposition temperature | : | No data available |
| Upper and lower flammability or explosive limit | : | No data available, Not applicable |
| Vapour pressure | : | No data available |
| Relative vapour density at 20°C | : | No data available |
| Relative density | : | 1 – 1,05 |
| Solubility | : | Soluble in water. |
| Partition coefficient n-octanol/water (Log Pow) | : | No data available |
| Viscosity, kinematic | : | No data available |
| Viscosity, dynamic | : | No data available |
| Explosive properties | : | Product is not explosive. |
| Explosive limits | : | No data available |

9.2. Other information

No additional information available

SECTION 10: Stability and reactivity

| Reactivity | : | The product is non-reactive under normal conditions of use, storage and transport. |
| --- | --- | --- |
| Chemical stability | : | Stable under normal conditions. |
| Possibility of hazardous reactions | : | No dangerous reactions known under normal conditions of use. |
| Conditions to avoid | : | None under recommended storage and handling conditions (see section 7). |
| Incompatible materials | : | Strong oxidizing agents. Strong reducing agents. |
| Hazardous decomposition products | : | Under normal conditions of storage and use, hazardous decomposition products should not be produced. |
| Hardening time: | : | No additional information available |

SECTION 11: Toxicological information

11.1. Information on toxicological effects

| Acute toxicity (oral) | : | No data available |
| --- | --- | --- |
| Acute toxicity (dermal) | : | No data available |
| Acute toxicity (inhalation) | : | No data available |

| Alkyl Glucoside (68515-73-1) |  |
| --- | --- |
| LD50 oral rat | > 5000 mg/kg bodyweight Animal: rat, Guideline: OECD Guideline 423 (Acute Oral toxicity - Acute Toxic Class Method), Guideline: EU Method B.1 tris (Acute Oral Toxicity - Acute Toxic Class Method) |
| LD50 dermal rabbit | > 2000 mg/kg bodyweight Animal: rabbit, Guideline: OECD Guideline 402 (Acute Dermal Toxicity) |

| N,N-dimethyldodecylamine-N-oxide (1643-20-5) |  |
| --- | --- |
| LD50 oral rat | 1064 mg/kg bodyweight (OECD 401: Acute Oral Toxicity, Rat, Male / female, Experimental value, Oral, 14 day(s)) |
| LD50 dermal rat | > 2000 mg/kg bodyweight Animal: rat, Guideline: OECD Guideline 402 (Acute Dermal Toxicity), Guideline: EU Method B.3 (Acute Toxicity (Dermal)) |
| ATE CA (oral) | 1064 mg/kg bodyweight |

| N,N-dimethyltetradecylamine N-oxide (3332-27-2) |  |
| --- | --- |
| LD50 oral rat | > 1495 mg/kg bodyweight (OECD 401: Acute Oral Toxicity, Rat, Male / female, Experimental value, Oral, 14 day(s)) |
| LD50 dermal rat | > 2000 mg/kg bodyweight (EU Method B.3: Acute toxicity (dermal), 24 h, Rat, Male / female, Experimental value, Dermal, 14 day(s)) |
| ATE CA (oral) | 500 mg/kg bodyweight |

| Citric acid (77-92-9) |  |
| --- | --- |
| LD50 oral rat | 3000 – 5000 mg/kg |
| LD50 oral | 5400 mg/kg bodyweight (Equivalent or similar to OECD 401, Mouse, Male / female, Experimental value, Oral, 10 day(s)) |
| LD50 dermal rat | > 2000 mg/kg bodyweight (OECD 402: Acute Dermal Toxicity, 24 h, Rat, Male / female, Experimental value, Dermal, 14 day(s)) |

| Sodium hydroxide (1310-73-2) |  |
| --- | --- |
| LD50 dermal rabbit | 1350 mg/kg |
| ATE CA (Dermal) | 1350 mg/kg bodyweight |

| Skin corrosion/irritation | : | Causes skin irritation.pH: 6 – 8 |
| --- | --- | --- |

| Serious eye damage/irritation | : | Causes serious eye damage.pH: 6 – 8 |
| --- | --- | --- |

| Respiratory or skin sensitization | : | No data available |
| --- | --- | --- |

| Germ cell mutagenicity | : | No data available |
| --- | --- | --- |

| Carcinogenicity | : | No data available |
| --- | --- | --- |

| Reproductive toxicity | : | No data available |
| --- | --- | --- |

| STOT-single exposure | : | No data available |
| --- | --- | --- |

| STOT-repeated exposure | : | No data available |
| --- | --- | --- |

| Alkyl Glucoside (68515-73-1) |  |
| --- | --- |
| NOAEL (oral, rat, 90 days) | 100 mg/kg bodyweight Animal: rat, Guideline: EU Method B.26 (Sub-Chronic Oral Toxicity Test: Repeated Dose 90-Day Oral Toxicity Study in Rodents) |

| N,N-dimethyldodecylamine-N-oxide (1643-20-5) |  |
| --- | --- |
| NOAEL (oral, rat, 90 days) | 40 mg/kg bodyweight Animal: rat, Guideline: OECD Guideline 422 (Combined Repeated Dose Toxicity Study with the Reproduction / Developmental Toxicity Screening Test), Guideline: other: |

| N,N-dimethyltetradecylamine N-oxide (3332-27-2) |  |
| --- | --- |
| NOAEL (oral, rat, 90 days) | 40 mg/kg bodyweight Animal: rat, Guideline: OECD Guideline 422 (Combined Repeated Dose Toxicity Study with the Reproduction / Developmental Toxicity Screening Test) |

| Citric acid (77-92-9) |  |
| --- | --- |
| LOAEL (oral, rat, 90 days) | 8000 mg/kg bodyweight Animal: rat |
| NOAEL (oral, rat, 90 days) | 4000 mg/kg bodyweight Animal: rat |

| Aspiration hazard | : | No data available |
| --- | --- | --- |

| ALL PURPOSE CLEAN NATURAL |  |
| --- | --- |
| Viscosity, kinematic | No data available |

| Likely routes of exposure | : | Skin and eyes contact. Inhalation. |
| --- | --- | --- |
| Expected Symptoms/Effects, Acute and Delayed | : | May cause dermatitis, eye irritation, corneal oedema and chemical burns. Repeated or prolonged contact may cause skin irritation. |
| Symptoms/effects after inhalation | : | None under normal conditions. |
| Symptoms/effects after skin contact | : | Irritation. |
| Symptoms/effects after eye contact | : | Serious damage to eyes. |
| Symptoms/effects after ingestion | : | None under normal conditions. |
| Chronic symptoms | : | No effects known. |

SECTION 12: Ecological information

12.1. Toxicity

| Ecology - general | : | The product is not considered harmful to aquatic organisms nor to cause long-term adverse effects in the environment. |  |  |
| --- | --- | --- | --- | --- |
| Hazardous to the aquatic environment, short–term (acute) |  |  | : | Not classified. |
| Hazardous to the aquatic environment, long–term (chronic) |  |  | : | Not classified. |

| ALL PURPOSE CLEAN NATURAL |  |
| --- | --- |
| Partition coefficient n-octanol/water (Log Pow) | No data available |

| Alkyl Glucoside (68515-73-1) |  |
| --- | --- |
| LC50 - Fish [1] | 100,81 mg/l Test organisms (species): Danio rerio (previous name: Brachydanio rerio) |
| LC50 - Fish [2] | 170 mg/l Test organisms (species): Danio rerio (previous name: Brachydanio rerio) |
| EC50 - Crustacea [1] | > 100 mg/l Test organisms (species): Daphnia magna |
| EC50 72h - Algae [1] | 27,22 mg/l Test organisms (species): Desmodesmus subspicatus (previous name: Scenedesmus subspicatus) |
| EC50 72h - Algae [2] | 37 mg/l Test organisms (species): Desmodesmus subspicatus (previous name: Scenedesmus subspicatus) |
| Organic Carbon Normalized Adsorption Coefficient (Log Koc) | 1,7 OECD 121 |

| N,N-dimethyldodecylamine-N-oxide (1643-20-5) |  |
| --- | --- |
| LC50 - Fish [1] | 134 mg/l (OECD 203: Fish, Acute Toxicity Test, 96 h, Danio rerio, Semi-static system, Fresh water, Experimental value) |
| LC50 - Fish [2] | 31,8 mg/l Test organisms (species): Danio rerio (previous name: Brachydanio rerio) |
| EC50 - Crustacea [1] | 3,9 mg/l (OECD 202: Daphnia sp. Acute Immobilisation Test, 48 h, Daphnia magna, Static system, Fresh water, Experimental value, Locomotor effect) |
| EC50 - Crustacea [2] | 3,1 mg/l Test organisms (species): Daphnia magna |
| ErC50 algae | 0,081 mg/l Source: NITE |
| NOEC (chronic) | 0,7 mg/l Test organisms (species): Daphnia magna Duration: '21 d' |
| Partition coefficient n-octanol/water (Log Pow) | < 2,7 (Calculated) |

| N,N-dimethyltetradecylamine N-oxide (3332-27-2) |  |
| --- | --- |
| LC50 - Fish [1] | 0,16 mg/l Source: Ecological Structure Activity Relationships |
| LC50 - Fish [2] | 2,4 mg/l Test organisms (species): Danio rerio (previous name: Brachydanio rerio) |
| EC50 - Crustacea [1] | 0,086 mg/l Source: Ecological Structure Activity Relationships |
| EC50 96h - Algae [1] | 0,061 mg/l Source: Ecological Structure Activity Relationships |
| NOEC chronic fish | 0,42 mg/l Test organisms (species): Pimephales promelas Duration: '302 d' |
| NOEC (chronic) | 0,7 mg/l Test organisms (species): Daphnia magna Duration: '21 d' |
| Partition coefficient n-octanol/water (Log Pow) | 5,66 Source: Chemical Substances Hazard Assessment Report/Initial Risk Assessment Report |

| Citric acid (77-92-9) |  |
| --- | --- |
| LC50 - Fish [1] | 440 – 760 mg/l (Equivalent or similar to OECD 203, 48 h, Leuciscus idus, Static system, Fresh water, Experimental value, Nominal concentration) |
| LC50 - Other aquatic organisms [1] | > 10 mg/l Bacteria |
| Partition coefficient n-octanol/water (Log Pow) | -1,8 – -1,55 (Experimental value) |
| Organic Carbon Normalized Adsorption Coefficient (Log Koc) | 1 (log Koc, SRC PCKOCWIN v2.0, Calculated value) |

12.2. Persistence and degradability

| ALL PURPOSE CLEAN NATURAL |  |
| --- | --- |
| Persistence and degradability | Expected to be readily biodegradable based on ingredients. |

| Alkyl Glucoside (68515-73-1) |  |
| --- | --- |
| Persistence and degradability | Readily biodegradable in water. |

| N,N-dimethyldodecylamine-N-oxide (1643-20-5) |  |
| --- | --- |
| Persistence and degradability | Readily biodegradable in water. |

| Citric acid (77-92-9) |  |
| --- | --- |
| Persistence and degradability | Biodegradable in the soil. Readily biodegradable in water. |
| Biochemical oxygen demand (BOD) | 0,42 g O₂/g substance |
| Chemical oxygen demand (COD) | 0,728 g O₂/g substance |
| ThOD | 0,686 g O₂/g substance |

12.3. Bioaccumulative potential

| ALL PURPOSE CLEAN NATURAL |  |
| --- | --- |
| Bioaccumulative potential | No test data of component(s) available. |
| Partition coefficient n-octanol/water (Log Pow) | No data available |

| Alkyl Glucoside (68515-73-1) |  |
| --- | --- |
| Bioaccumulative potential | Log Kow <1.77. |
| Organic Carbon Normalized Adsorption Coefficient (Log Koc) | 1,7 OECD 121 |

| N,N-dimethyldodecylamine-N-oxide (1643-20-5) |  |
| --- | --- |
| Bioaccumulative potential | Low potential for bioaccumulation (Log Kow < 4). |
| Partition coefficient n-octanol/water (Log Pow) | < 2,7 (Calculated) |

| N,N-dimethyltetradecylamine N-oxide (3332-27-2) |  |
| --- | --- |
| Bioaccumulative potential | No test data available. |
| Partition coefficient n-octanol/water (Log Pow) | 5,66 Source: Chemical Substances Hazard Assessment Report/Initial Risk Assessment Report |

| Citric acid (77-92-9) |  |
| --- | --- |
| Bioaccumulative potential | Not bioaccumulative. |
| Partition coefficient n-octanol/water (Log Pow) | -1,8 – -1,55 (Experimental value) |
| Organic Carbon Normalized Adsorption Coefficient (Log Koc) | 1 (log Koc, SRC PCKOCWIN v2.0, Calculated value) |

12.4. Mobility in soil

| ALL PURPOSE CLEAN NATURAL |  |
| --- | --- |
| Ecology - soil | No (test) data on mobility of the substance available. |
| Partition coefficient n-octanol/water (Log Pow) | No data available |

| Alkyl Glucoside (68515-73-1) |  |
| --- | --- |
| Mobility in soil | 0,2624 Source: EPISUITE |
| Organic Carbon Normalized Adsorption Coefficient (Log Koc) | 1,7 OECD 121 |

| N,N-dimethyldodecylamine-N-oxide (1643-20-5) |  |
| --- | --- |
| Ecology - soil | Low potential for adsorption in soil. |
| Partition coefficient n-octanol/water (Log Pow) | < 2,7 (Calculated) |

| N,N-dimethyltetradecylamine N-oxide (3332-27-2) |  |
| --- | --- |
| Mobility in soil | 3,99 Source: Quantitative Structure Activity Relation |
| Surface tension | 32,4 – 32,5 mN/m (21 °C, 1.04 g/l, EU Method A.5: Surface tension) |
| Partition coefficient n-octanol/water (Log Pow) | 5,66 Source: Chemical Substances Hazard Assessment Report/Initial Risk Assessment Report |

| Citric acid (77-92-9) |  |
| --- | --- |
| Surface tension | No data available in the literature |
| Ecology - soil | Highly mobile in soil. |
| Organic Carbon Normalized Adsorption Coefficient (Log Koc) | 1 (log Koc, SRC PCKOCWIN v2.0, Calculated value) |
| Partition coefficient n-octanol/water (Log Pow) | -1,8 – -1,55 (Experimental value) |

12.5. Other adverse effects

| Ozone | : | No data available |
| --- | --- | --- |

SECTION 13: Disposal considerations

13.1. Disposal methods

| Regional waste regulation | : | Disposal must be done according to official regulations. |
| --- | --- | --- |
| Waste treatment methods | : | Dispose of contents and or container in accordance with licensed collector’s sorting instructions. |
| Sewage disposal recommendations | : | Disposal must be done according to official regulations. |
| Product/Packaging disposal recommendations | : | Disposal must be done according to official regulations. |
| Additional information | : | Do not re-use empty containers. |
| Ecological waste information | : | Avoid release to the environment. |

SECTION 14: Transport information

| TDG | IMDG | IATA |
| --- | --- | --- |
| 14.1. UN number |  |  |
| Not regulated for transport |  |  |
| 14.2. Proper Shipping Name |  |  |
| Not applicable | Not applicable | Not applicable |
| Transport document description |  |  |
| Not applicable | Not applicable | Not applicable |
| 14.3. Transport hazard class(es) |  |  |
| Not applicable | Not applicable | Not applicable |
| 14.4. Packing group |  |  |
| Not applicable | Not applicable | Not applicable |
| 14.5. Environmental hazards |  |  |
| Not applicable | Not applicable | Not applicable |
| No supplementary information available |  |  |

14.6. Special precautions for user

TDG, IMDG, IATA

No data available

14.7. Transport in bulk according to Annex II of MARPOL 73/78 and the IBC Code

Not applicable

SECTION 15: Regulatory information

15.1. National regulations

Alkyl Glucoside (68515-73-1)

Listed on the Canadian DSL (Domestic Substances List)

N,N-dimethyldodecylamine-N-oxide (1643-20-5)

Listed on the Canadian DSL (Domestic Substances List)

N,N-dimethyltetradecylamine N-oxide (3332-27-2)

Listed on the Canadian DSL (Domestic Substances List)

Glycolipids, sophorese-contg, Stamerella bombicola-fermented, from cooking oil wastes and sucrose

Not listed on the Canadian DSL (Domestic Substances List)/NDSL (Non-Domestic Substances List)

Citric acid (77-92-9)

Listed on the Canadian DSL (Domestic Substances List)

Sodium hydroxide (1310-73-2)

Listed on the Canadian DSL (Domestic Substances List)

15.2. International regulations

Alkyl Glucoside (68515-73-1)

Listed on the United States TSCA (Toxic Substances Control Act) inventory - Status: Active

N,N-dimethyldodecylamine-N-oxide (1643-20-5)

Listed on the United States TSCA (Toxic Substances Control Act) inventory - Status: Active

N,N-dimethyltetradecylamine N-oxide (3332-27-2)

Listed on the United States TSCA (Toxic Substances Control Act) inventory - Status: Active

Glycolipids, sophorese-contg, Stamerella bombicola-fermented, from cooking oil wastes and sucrose

Not listed on the United States TSCA (Toxic Substances Control Act) inventory

Citric acid (77-92-9)

Listed on the United States TSCA (Toxic Substances Control Act) inventory - Status: Active Listed on INSQ (Mexican National Inventory of Chemical Substances)

Sodium hydroxide (1310-73-2)

Listed on the United States TSCA (Toxic Substances Control Act) inventory - Status: Active Listed on INSQ (Mexican National Inventory of Chemical Substances)

SECTION 16: Other information

| Issue date | : | 07-05-2025 |
| --- | --- | --- |

| Full text of hazard classes and H-statements: |  |
| --- | --- |
| H315 | Causes skin irritation |
| H318 | Causes serious eye damage |

It is the responsibility of the user to provide a safe workplace, using the health and safety information contained herein as a guide. Project Clean Inc. will accept no liability for damages or loss incurred from the improper handling and use of this product.
The information provided in the Safety Data Sheet has been obtained from current sources and is believed to be reliable.